Verification Document – Frame Aligner Project

Verification Document – Frame Aligner Project

1.1 Purpose

The purpose of this verification document is to define the strategy, environment, and test plan used to verify the Frame Aligner module.
The goal of the verification process is to ensure that the design correctly detects frame alignment sequences, handles both valid and invalid headers, and manages the transition into and out of the alignment state according to the functional specification.

1.2 DUT Description

The Frame Aligner is a hardware module responsible for monitoring an incoming stream of serial data divided into frames of 96 bits (12 bytes) each.
Each frame is structured as follows:

- 2 bytes (16 bits) — Header

- 10 bytes (80 bits) — Payload

The module recognizes two types of valid headers (e.g., 0xAFAA and 0xBA55), which signal the start of a valid data payload.
To achieve synchronization (alignment), the Frame Aligner must detect three consecutive valid frames (each with a valid header).
Once aligned, it asserts the internal signal alignment = 1 and continues to process payload data.

If, after entering the aligned state, four consecutive invalid headers are detected, the module automatically exits alignment (alignment = 0).
This mechanism ensures reliable synchronization on valid data sequences while providing resilience against bit errors or invalid frame patterns.

1.3 Verification Objective

The verification objective is to ensure that:

- The module detects valid headers and aligns only after three consecutive valid frames.

- The module correctly deasserts alignment after four invalid headers during aligned state.

- All internal states (FSM transitions) are exercised and behave as expected.

- Payload and header integrity are verified under different traffic conditions, including random and directed tests.

2. Verification Environment

2.1 Overview

The verification environment for the Frame Aligner module is a structured, class-based SystemVerilog environment designed to thoroughly validate the DUT’s functionality.
The environment includes components for stimulus generation, driving and monitoring of interfaces, functional checking, and coverage collection.
The testbench is modular and reusable, allowing different test configurations (directed, random, error injection) to be executed efficiently.

The top-level module (top.sv) connects the DUT (frame_aligner) with the verification environment through virtual interfaces and mailboxes, ensuring synchronization and communication between components.

2.2 Testbench Architecture

2.2.1 inf.sv

Defines the input interface (inf_in) that connects the verification environment to the DUT inputs.
It contains all the required signals to drive the frame stream:
clk, reset, rx_data (8-bit bus), and additional control signals.
This interface serves as the main connection point for both the driver and the monitor_in to the DUT.

2.2.2 inf_scb.sv

Defines the scoreboard interface (inf_scb) used for observation of DUT outputs such as frame_detect, fr_byte_position, alignment status, and counters.
This interface provides visibility to the monitor_out and scoreboard, enabling comparison between expected and actual DUT behavior.

2.2.3 transaction_in

Implements the class transaction_in, which represents the structure of an input transaction.
Key fields include:

- header_type, header, and payload [79:0].

- Contains randomization constraints controlling header distribution (HEAD_1, HEAD_2, ILLEGAL, etc.).

- The post_randomize() function assigns the correct header value based on the selected header type.

2.2.4 transaction_out

Implements the class transaction_out, which represents the DUT output transaction.
It stores output data such as frame detection results, alignment indication, and frame byte position.
This class is used by the monitors and scoreboard to analyze DUT responses.

2.2.5 generator

The generator is responsible for creating a sequence of input transactions (transaction_in) according to the selected test scenario.

- Transactions are sent to the driver via a mailbox (gen2drv).

- Supports multiple test modes defined in test_mode_t (e.g., TEST_RANDOM, TEST_DIRECTED, TEST_HEADER_IN_PAYLOAD).

- At the end of sequence generation, the generator triggers the ended event to notify the test that stimulus creation is complete.

2.2.6 driver

The driver converts high-level transactions into cycle-accurate signal activity on the DUT input interface (inf_in).

- On each clock cycle, the driver transmits one byte (8 bits) of data, dividing the full 96-bit frame into 12 consecutive bytes (2-byte header + 10-byte payload).

- This byte-streaming behavior provides realistic protocol timing and accurately models serial data flow toward the DUT.

2.2.7 monitor_in

The input monitor observes the signals on the DUT input interface (inf_in) and reconstructs the transactions that were actually driven.
It sends these observed transactions to the scoreboard to ensure proper data reception by the DUT.
It also contributes to functional coverage related to input frames and header types.

2.2.8 monitor_out

The output monitor observes the DUT output interface (inf_out) and captures signals such as frame_detect, and fr_byte_position.
It forwards this data to the scoreboard for comparison with the reference model’s predicted behavior.

2.2.9 scoreboard

The scoreboard is the central checking component of the environment and includes the reference model.

- It models the expected Frame Aligner behavior:

  - Alignment is achieved only after three consecutive valid headers.

  - Alignment is lost after four consecutive invalid headers while aligned.

- Compares DUT outputs with predicted results and tracks counters like good_cnt, bad_cnt, and payload_wait.

- Includes covergroups for FSM states, header sequences, and alignment transitions.

- Reports mismatches and coverage results at the end of simulation.

2.2.10 environment

Implements the environment class — the main container of the verification testbench.

- Instantiates and connects all major components: generator, driver, monitors, scoreboard, and coverage.

- Connects mailboxes and virtual interfaces between components.

- Manages simulation phases: build, run, check, and report.

- Controls test flow, synchronization events, and end-of-test signaling.

2.2.11 test

Defines the test program, which configures and runs a specific verification scenario.

- Selects the desired test_mode and sets parameters such as repeat_count.

- Instantiates the environment and triggers its execution.

- Waits for the ended event before completing simulation and collecting coverage reports.

2.2.12 top

The top-level module that ties together the DUT and the verification environment.

- Generates clk and reset signals.

- Instantiates all interfaces (inf_in, inf_out, inf_scb).

- Creates the test object and starts simulation.

3. Test Plan

3.1 Objectives

The goal of the test plan is to guarantee that the Frame Aligner module:

Correctly performs alignment and de-alignment based on the header sequence.

Properly handles valid and invalid headers according to the functional specification.

Maintains payload integrity under both deterministic and randomized traffic conditions.

Accurately tracks its internal frame-byte position (fr_byte_position), ensuring that at any time the DUT knows its exact location within the 12-byte frame (2-byte header + 10-byte payload).

Each test case is designed to exercise different aspects of functionality, timing, corner cases, and error conditions, including the verification of byte-position tracking throughout alignment and data processing.

3.2 Test Descriptions

| # | Test Name | Test Mode / ID | Purpose / Objective | Stimulus Description | Expected Behavior / Pass Criteria |
| --- | --- | --- | --- | --- | --- |
| 1 | Random Frames Test | TEST_RANDOM | Validate random operation of DUT under arbitrary headers and payloads. | Randomized transaction_in with random headers and payloads. | frame_detect asserts after 3 valid headers and drops after 4 invalid. fr_byte_position cycles 0→11 per frame. No mismatches. |
| 2 | Basic Directed Sequence | TEST_DIRECTED | Deterministic pattern to verify alignment and de-alignment across valid and illegal headers. | direct_gen = 5×HEAD_1, 4×ILLEGAL, 5×HEAD_2. | Alignment asserts after 3×HEAD_1, drops on ILLEGAL, re-asserts on HEAD_2. Byte counter resets per frame. |
| 3 | Header Variation Set 1 | TEST_DIRECTED_2 | Check handling of custom headers HEAD_TEST1-4. | Sequence of {HEAD_TEST1, HEAD_TEST2, HEAD_TEST3, HEAD_TEST4}. | All headers decoded correctly; no false alignment; fr_byte_position sequential. |
| 4 | Mixed Valid/Illegal Sequence | TEST_DIRECTED_3 | Verify alignment loss and re-entry. | Pattern = {HEAD_1×4, ILLEGAL, HEAD_2×2, ILLEGAL}. | Alignment asserts after HEAD_1 run, drops on ILLEGAL, re-aligns on HEAD_2. fr_byte_position resets correctly. |
| 5 | Header in Payload Test | TEST_HEADER_IN_PAYLOAD | Ensure payload containing header-like values (0xAFAA / 0xBA55) does not cause false alignment. | Payloads intentionally include 0xAA, 0xAF pattern at byte 4. | frame_detect and alignment unchanged; fr_byte_position increments normally. |
| 6 | Mixed Header Types Set 2 | TEST_DIRECTED_4 | Validate interaction between standard and test headers. | Pattern = {HEAD_1, HEAD_1, HEAD_TEST1, HEAD_2, HEAD_2, HEAD_TEST2}. | All frames decoded properly; no early alignment or counter errors. |
| 7 | Complex Header/Illegal Pattern | TEST_DIRECTED_5 | Stress-test alignment stability under long mixed sequence. | {HEAD_1×3, ILLEGAL×3, HEAD_TEST2, HEAD_2×3, ILLEGAL×3, HEAD_TEST2}. | Alignment toggles only per specification; fr_byte_position stable across frames. |
| 8 | Extended Header Set | TEST_DIRECTED_6 | Confirm support for HEAD_TEST1-6 headers. | {HEAD_TEST1-HEAD_TEST6}. | All headers recognized; alignment and frame_detect consistent. |
| 9 | Header Edge Cases | TEST_HEADER_EDGE_CASES | Check boundary byte values (0x00/0xFF in MSB/LSB). | Headers = 0x00AA, 0xAA00, 0xFFAA, 0xAAFF. | No false alignment; proper header decode; fr_byte_position wraps correctly. |
| 10 | New Directed Pattern | TEST_DIRECTED_7 | Validate behavior for latest header types HEAD_TEST7 & HEAD_TEST8 and their repetition. | Pattern = {HEAD_TEST7, HEAD_TEST7, HEAD_TEST8}. | DUT handles new header values correctly; no FSM violations; coverage bins for HEAD_TEST7/8 hit. |

3.3 Header Type Definition

| Header Name | Hex Value | Description / Purpose | Validity |
| --- | --- | --- | --- |
| HEAD_1 | 16'hAFAA | Primary valid header type used for frame alignment detection. | Valid |
| HEAD_2 | 16'hBA55 | Secondary valid header type used for alternate frame recognition. | Valid |
| HEAD_TEST1 | 16'hAF55 | Used in directed tests to verify correct differentiation between similar headers (AFAA vs AF55). | Test Header |
| HEAD_TEST2 | 16'hBAAA | Tests recognition of headers differing in lower byte from valid types. | Test Header |
| HEAD_TEST3 | 16'hAF00 | Checks header detection when low byte = 0x00 (edge-case LSB). | Test Header |
| HEAD_TEST4 | 16'hBA00 | Similar to HEAD_TEST3 but based on BA family; tests zero LSB case. | Test Header |
| HEAD_TEST5 | 16'h00AF | Tests zero MSB scenario in AF family header pattern. | Test Header |
| HEAD_TEST6 | 16'h00BA | Tests zero MSB scenario in BA family header pattern. | Test Header |
| HEAD_TEST7 | 16'hAAAF | Added for extended testing; verifies new alignment edge pattern (mirrored byte order). | Test Header |
| HEAD_TEST8 | 16'h55BA | Added for extended testing; tests reversed BA55 byte pattern for header symmetry behavior. | Test Header |
| ILLEGAL | Randomized value (≠ AFAA, BA55) | Used to test invalid header handling and de-alignment recovery. | Invalid |

4. Coverage

4.1 Objectives

The goal of functional and code coverage in the Frame Aligner verification environment is to:

- Quantify how thoroughly the DUT’s functionality has been exercised.

- Ensure that all FSM states, header types, alignment transitions, and corner-case behaviors are covered.

- Confirm correct progression of the internal counter fr_byte_position throughout all frame phases.

- Verify that the output signal frame_detect asserts (1) exactly after three consecutive valid headers and de-asserts when alignment is lost.

- Demonstrate proper synchronization between frame_detect, alignment status, and byte-position tracking.

Coverage results are used to drive regression closure and to prove that the verification environment has achieved measurable completeness.

4.2 Functional Coverage Definition

| Covergroup Name | Scope / Signals Sampled | Coverage Intent / Bins | Sampling Event |
| --- | --- | --- | --- |
| cg_headers | header_type (HEAD_1, HEAD_2, HEAD_TEST1–7, ILLEGAL) | Ensure all header types are generated and decoded correctly. | Sampled in monitor_in after each header transmission. |
| cg_alignment_transitions | alignment, valid_header_count, invalid_header_count | Cover all alignment assert/de-assert transitions (e.g., Idle → Aligned → Lost). Cross-bins for transition sequences. | Sampled in scoreboard whenever alignment changes. |
| cg_frame_detect | frame_detect, valid_header_count, alignment | Verify that frame_detect asserts exactly after three consecutive valid headers and de-asserts when alignment is lost. Cross coverage between frame_detect events and alignment state ensures proper synchronization. | Sampled on every frame_detect transition (rise/fall) within monitor_out or scoreboard. |
| cg_fsm_states | FSM state (FR_IDLE, FR_HLSB, FR_HMSB, FR_DATA) | Confirm all FSM states and transitions are exercised. Cross-bins for valid state sequences. | Sampled each clock when state ≠ previous state. |
| cg_illegal_header | header_type == ILLEGAL | Verify illegal headers are generated and handled correctly (de-alignment or ignore). | Sampled in monitor_in whenever an ILLEGAL header is detected. |
| cg_byte_position | fr_byte_position (0 → 11) | Validate byte-position counter increments sequentially and resets correctly per frame. Cross bins with alignment. | Sampled once per byte received. |

5. Sign-Off Criteria

5.1 Pass / Fail Criteria

- A test is considered PASSED when all the following conditions are met:

- No functional mismatches are reported by the scoreboard or reference model

- No assertion failures, simulation errors, or unexpected warnings occur.

- All expected signals behave according to specification:

- frame_detect asserts exactly after three consecutive valid headers and de-asserts when alignment is lost.

- fr_byte_position increments sequentially from 0 → 11 for every frame and resets at frame boundaries.

- Alignment enters and exits states correctly according to FSM transitions

- Functional coverage and code coverage thresholds (see Section 5.2) are reached or exceeded.

- A test is considered FAILED if any of the following occur:

- Scoreboard mismatch between expected and actual DUT outputs.

- Assertion or simulation failure.

- Coverage below target thresholds.

- Any persistent or non-deterministic behavior in alignment, frame_detect, or fr_byte_position.

5.2 Coverage Closure Targets

| Category | Target (%) | Description |
| --- | --- | --- |
| Functional Coverage | ≥ 95 % | All covergroups (headers, FSM states, alignment transitions, frame_detect, fr_byte_position) must meet their coverage goals. |
| FSM State Coverage | ≥ 95 % | Every Frame Aligner FSM state and transition exercised at least once. |
| Code Coverage | ≥ 90 % | Combined line, branch, condition, and toggle coverage for RTL. |
| Header × Alignment Cross | 100 % | Each header type seen in both aligned and unaligned modes. |
| Byte-Position Tracking | 100 % | All positions (0–11) hit in both aligned and unaligned modes. |
| frame_detect Behavior | 100 % | All transitions observed and validated relative to valid/invalid header sequences. |

6. Bug Tracking and Known Issues

6.1 Bug #1 — Incorrect fr_byte_position Update During Partial Header

Detected by: Waveform inspection (fr_byte_position trace in Verdi)

During simulation, it was observed that the internal counter fr_byte_position increments to 1 after receiving only the first byte (LSB) of the header, instead of waiting until the complete 2-byte header is received.
This causes the byte position to advance prematurely, potentially misaligning the payload indexing and affecting synchronization logic.

Expected Behavior:

fr_byte_position should remain 0 throughout both header bytes (FR_HLSB and FR_HMSB states)
and only increment to 1 after the full 2-byte header has been correctly received and validated.